%N tsysmed

%D Replace the system temperatures by a median smoothed curve

%P gmx

%: calibration

%B

TSYSMED reads in the x and y system temperatures, and fits

a median smooted curve to them. It does not check for changes

in position, so this taks should only be used on one position

data sets. It also only changes xtsys and ytsys, and does not

touch systemp. This means that after using this task, systemp

should not be used (at least it will be inconsistent with

xtsys and ytsys).

It is a changed version of the task UVTCOR

%A vis

Input visibility file. No default.

%A out

Output visibility file.  No default.