%N plboot

%D Set the flux scale of a visibility dataset given a planet obs.

%P rjs

%: calibration

%B

PlBoot is a MIRIAD program which corrects the flux scale in

visibility datasets. In doing this, it assumes that the flux

scale is out by a constant factor. PlBoot looks for observations

of planets, and given its model of the planetary visibility

function, it computes the factor needed to correct the dataset's

flux scale.

To fix the flux scale, PlBoot modifies calibration tables attached

to each dataset.

%A vis

Input visibility datasets. Several datasets can be given (wildcards

are supported). The datasets should include observations of a planet.

%A select

Normal uv-selection parameter. This selects the data in the input

datasets to analyse. See the help on ``select'' for more information.

plboot will use any data that has a source name which it recognises

as a planet. You may wish to select just the shortest spacing, where

the planet is strongest.

%A device

PGPLOT device to plot the planet model data as well as the visibility

data. The default is to not produce a plot.

%A options

Extra processing options. Several can be given, separated by commas.

vector  Use the real part (rather than amplitude) of the data and

model. This option should be used if the visibility data are

phase calibrated

noapply Do not apply the scale factor (just evaluate it).

nofqav  When dealing with amplitude data, PLBOOT normally

averaging in frequency first, to avoid noise

biases. The nofqav disables this averaging.