%N moslst

%D LST coverage of the different pointings in a mosaic.

%P rjs

%: uv analysis

%B

Determine the LSTs at which particular fields in a mosaic

run are observed.

%A vis

Input vis datasets. No default.

%A fields

Indices of the pointings to check. If nothing is given, all

pointings are checked.

%A source

Base source name of the mosaic. By default, MOSLST assumes that

any source whose names ends in digits is a pointing of a mosaic,

and takes this digits as the pointing number. If the SOURCE parameter

is given, then MOSLST takes only those sources starting with that

name, ending in digits, and with a possible underscore character

in between.

%A device

PGPLOT device for the LST ranges. The default is no plot.

%A ppp

"Pointings per plot". This is used for plotting, and allows the

output to be broken into a number of subplots. Each subplot will

have (at most) the given number of pointings in it. The default

is to not break it into subplots.

%A log

Output log file. Default is the terminal.

%A gap

Gap tolerance. This is used for the log output to indicate

large gaps in the LST coverage. The default is 36 minutes.