%N tpgains

%D Estimate antenna gains from total power measurements.

%P mchw

%: tpower, antenna gains

%A vis

Name of input visibility data file. No default.

%A again

`again' gives an antenna amplitude factor as a function

again * sqrt(total_power/systemp)

Default value 0. gives no amplitude  change.

%A rgain

`rgain' gives an antenna amplitude factor as a function

rgain * sqrt(systemp/total_power)

Default value 0. gives no amplitude  change.

%A pgain

`pgain' gives the antenna phase change with total power in

degrees/Kelvin. Default value is 0. i.e. No phase change.

Typical value at 100 GHz is pgain = -280 degrees/K.

%A scale

Scale total power to tsys [K]. Default no scaling applied.

Three values for each antenna: tpower = a + b*tpower + c*tpower**2

These values can be obtained by editing the log file from VARFIT

E.g. varfit vis=? xaxis=tpower refant=1 options=xscale log=varlog

The quadratic term is currently ignored.

%A refant

Compute the antenna gains relative to refant.

The gain of refant is set to cmplx(1.,0.). Default=0. no refant.

%A out

Name of output calibration file. The default is to write the gain

corrections into the input visibility file.