%N uvedit

%D Editing of the baseline of a UV data set.

%P pjt

%: calibration, uv-data

%B

UVEDIT is a MIRIAD task which allows baseline editing of a UV

data set.  As a result of the editing, certain header variables

of the data set are changed.  The headers `corr' and `wcorr'

are always changed since they are the data themselves.  The

headers `coord(2)' are the baseline coordinates and, as a

result, are also always changed (except with the option nouv).

The UV variable headers `lst', `ut', and `time' are updated

whenever a time offset is entered.  The headers `ra', `dec',

`obsra', and `obsdec' are changed whenever a positional

correction is entered with the ra or dec input.  Finally,

antenna coordinate corrections will cause the header `antpos'

to be corrected.

NOTE: There can be NO select keyword for this routine!  If

one includes the select option, then data that is not selected

will not be copied!

%A vis

The name of the input UV data set.  At least one file name must

be supplied.  Up to 50 visibility files are currently allowed.

%A source

The name of the source to apply corrections to if more than

one source is present in a UV data set.  All UV data that does

not correspond to the input source name is copied without being

edited.  If this keyword is not set (the default), then all

sources are edited.  Only one source name may be input.

Note: it probably doesn't make sense to use this keyword with

any other options except the ra and dec keywords.  If any other

editing (except ra/dec) is requested along with this keyword,

a warning message will be issued but the editing will proceed.

%A apfile

The name of a file that contains the absolute antenna positions

entered in increasing antenna order.  Only one antenna position

file is permitted.  The first line of the file is ALWAYS skipped

and the remaining lines MUST contain three entries corresponding

to the X, Y, and Z equatorial coordinates, respectively, in units

of nanoseconds.  The function of this keyword is identical to the

``antpos'' keyword except that there is no way to specify a

subset of antennae; every antenna up to and including the largest

antenna number must be present in the file.

NOTE: You may only specify at most one of the ``apfile'',

``antpos'', or ``dantpos'' keywords.

In the case of options=sma, the apfile is the original

antenna file which can be copied over from the original

MIR-formated data, e.g. 050604_03:44:03/antennas which

contains four entries corresponding to the antenna number,

the X, Y, Z geocentric cooridnates in unit of meter.

the reference antenna's X,Y,Z coordinates are specified

to be zeros. Any corrections for the antenna positions

must be given in the keyword smaoffset below. If no entries

are input from the keyword smaoffset, positions in the ``apfile''

are considered as the updated values and will be used to

correct for the vis data.

%A smaoffset

Inputs are the antenna coordinate offsets for SMA entered in

the following order (NO checking is done for consistency):

smaoffset = A1,X1,Y1,Z1,A2,X2,Y2,Z2,A3,X3,Y3,Z3,....

The input values are the antenna number and the three geocentric

coordinate offsets (entered in units of millimeter). These input

values are added to the original absolute coordinates read from

the antenna file given in keywork apfile.

Note that A1 does not necesarily have to correspond to Antenna 1;

it is used to represent the variable containing the antenna

number.  Antennas present in the data but not included in the

input value list are treated as having a zero coordinate offset.

%A antpos

Inputs are the absolute equatorial coordinates entered in the

following order (NO checking is done for consistency):

antpos = A1,X1,Y1,Z1,A2,X2,Y2,Z2,A3,X3,Y3,Z3,....

The input values are the antenna number and the three equatorial

coordinates (entered in units of nanoseconds).  Note that A1 does

not necesarily have to correspond to Antenna 1; it is used to

represent the variable containing the antenna number.  Antenna

(and the corresponding coordinates) not included in the input

listing do not have their coordinates changed.

NOTE: An antenna position value of zero is not possible.  If

an antenna value is set to zero, the current value from the

data is used.  To force an antenna position value to be zero,

use the keyword ``dantpos''.

NOTE: You may only specify at most one of the ``apfile'',

``antpos'', or ``dantpos'' keywords.

%A dantpos

Inputs are the equatorial coordinate offsets entered in the

following order (NO checking is done for consistency):

dantpos = A1,X1,Y1,Z1,A2,X2,Y2,Z2,A3,X3,Y3,Z3,....

The input values are the antenna number and the three equatorial

coordinate offsets (entered in units of nanoseconds).  These input

values are added to the absolute coordinates read from the data.

Note that A1 does not necesarily have to correspond to Antenna 1;

it is used to represent the variable containing the antenna

number.  Antenna present in the data but not included in the

input value list are treated as having a zero coordinate offset.

NOTE: You may only specify at most one of the ``apfile'',

``antpos'', or ``dantpos'' keywords.

NOTE: The dantpos keyword is the usually used when correcting antenna

position errors in a VLA observation. The  coordinate system used by

Miriad and the VLA are the same, and the baseline changes provided

by the VLAIS system need only be changed from units of meters to nanosec

when using uvedit (1 nanosec = 0.2997 meters).

%A ra

Input is either an absolute or delta right ascension of the

phase tracking center.  If one value is present, it is considered

as a offset position and is to be entered as time seconds.

Otherwise, three values are expected and are to be entered in

the following order:

ra = HH,MM,SS.S

The right ascension (offset) is relative to the epoch coordinates.

The default value is 0 seconds offset (no change).

%A dec

Input is either an absolute or delta declination of the phase

tracking center.  If only one value is present, it is considered

as a offset position and is to be entered in arcseconds.

Otherwise, three values are expected and are to be entered in

the following order:

dec = DD,MM,SS.S

The declination (offset) is relative to the epoch coordinates.

The default value is 0 arcseconds offset (no change).  If the

absolute declination is negative but the DD value is 0, then make

the MM value negative.  If MM is also 0, then make SS.S negative.

%A time

Input is a time offset (in seconds) to be added to the clock time.

The default value is 0 seconds offset (no change).

%A delay

Inputs are the delay error corrections for each antenna.

The inputs are the delay values for each antenna entered in the

following order (in units of nanoseconds):

delay = D1,D2,D3,....

If no value for delay is specified, the array of antenna values

are set to 0 nanoseconds offset.  Also, a resulting difference

(D[i]-D[j]) of less than 0.05 nanoseconds is ignored (no change).

If a difference exists, the digital wide band data will be

reconstructed.  All previously flagged narrow band data will

be ignored in the reconstruction.  Also, the two (2) end

channels of each window will be ignored in the reconstruction.

%A out

The name of the output visibility file.  This parameter is

ignored when more than one visibility file is given.  If no value

for ``out'' is given or more than one visibility file is input,

then the output file name(s) will be the same as the input file

name(s) but with an "_c" appended to the file name

(ie. "Vis = saturn,jupiter" will result in output files "saturn_c"

and "jupiter_c").

%A options

Task enrichment options.  Minimum match is active.

nouv     Do not recompute the uvw (coord(1..3)

This option should, in principle, only be used with

the delay correction;

all other corrections should recompute u,v and w

dra

Multiply the dra values by a cos(obsdec) correction.

This is used to correct the dra value in the uv

dataset for MINT data taken at Hat Creek before

11dec93.  Before that date, the 1/cos(obsdec)

correction was not applied to the dra in the grid

file, so that the pointing was incorrect (instead

of dra arcseconds offsets, the offsets were

dra*cos(obsdec) arcseconds).

NOTE:  The obsdec used is the "old" obsdec.  If there

is a correction in declination, this is NOT applied

in computing the cos(obsdec).

sma

Allows to use the SMA corrdinates system, i.e.

The geocentric coodinates in unit of meter

uvrotate Rotate uv-coordinates from current to standard epoch.

This is needed for BIMA data, or any data that keeps

their UV(W) in current epoch instead of the more common

standard epoch.

time     adjust the time to the start of an integration time,

instead of the middle. Hence an offset of -inttime/2 will be

applied. Unlike the TIME keyword described above, this will have

no other side effect. After this change 'ut' and 'time' should

agree again as they did erroneously in BIMA data and CARMA

data before Feb 2006.

Normally 'ut' and 'time' are the mid point of each integration.

Do not use this option unless you know what you are doing.