%N calfit

%D Fit polynomial to amp/phase data

%: calibration

%P pjt

%B

CALFIT is a MIRIAD task which fits polynomials to the wideband data

in a calibration dataset previously made by CALMAKE.

By default, CALFIT uses 2nd order for phases and 0th order for

amplitudes, without phase closure.  The resulting fit is stored in

various items in the calibration dataset, and can be over-written.

(See also DELHD, PUTHD, and COPYHD as MIRIAD system utilities)

When a 0th order fit is used for amplitude, CALFIT also displays the

gain factors (Jy/K) for each baseline, sideband and interval. The

dispersion of the phases from the least squares fit (radians) is also

reported.

%A gcal

The calibration dataset, as produced by CALMAKE. No default.

%A source

Names of the sources to which the fit is applied.

Default is to select all sources present in the calibration set.

%A order

The polynomial orders to be fit to the amplitude and phase. The

maximum order for both amplitude and phase is 10. Default: 0,2.

%A close

If set to true, a baseline based phase closure will be used in the

gain fits.

For 'true', breakpoints are forced to be equal across baselines,

taken from the first baseline found in the database.

Default: false

%A taver

Taver consists of two numbers, ``tgap'' and ``ttot'', both in minutes,

which are used for vector averaging. If the time interval between

any two successive data points is greater than ``tgap``, or if the

total time between the first data point in a vector average and any

succeeding data point exceeds ``ttot'', then a new vector average is

started.

Default: 0.0,1000.0 (no vector averaging)

%A mode

Fit 'phase' or 'amplitude'. If nothing specified, both

will be fit. Default: both.

%A write

A logical, which specifies whether to (over)write the calculated

polynomial coefficients. The item ``pdata'' is used for this.

This allows a ``dryrun'', only showing gains and phase fits.

Default: true.

%A units

Units to work in. Options are: K, K/Jy, Jy/K. See comments in CALFLAG.

Default: Jy/K.