%N sinbad

%D Calculate Tsys*(on-off)/off for autocorrelation data.

%P mchw

%: uv analysis

%B

SINBAD is a MIRIAD task to calculate (ON-OFF)/OFF*TSYS for

autocorrelation data. The preceeding OFF and TSYS record

for each antenna is used. When flags on the OFF scans are bad,

the scan is now ignored as an OFF scan.

Flags on the ON scans are copied to the output file.

Only valid ON scans are copied to the output file.

Missing TSYS can be replaced via the tsys= keyword as a last

resort, or the spectral window based systemp() UV variable

that should be present in normal MIRIAD datasets.

Normally the "on" UV-variable is used to find out which record

is the ON (on=1) or OFF (on=0) record. You can override this

by supplying two source names using the onoff= keyword

For baseline subtraction, see:  SINPOLY

For single dish mapping, see:  VARMAPS

%A vis

The name of the input autocorrelation data set.

Only one name must be given.

%A select

The normal uv selection commands. See the Users manual for details.

Warning(BUG):   do not use win() selection,the tsys values are still

not correctly obtained this way. Use UVCAT to preprocess your selection.

The default is to process all data.

%A line

The normal uv linetype in the form:

line,nchan,start,width,step

The default is all channels.

The output consists of spectral channels only.

%A out

The name of the output uv data set. No default.

%A tsys

Value for flat tsys spectrum if neither band average systemp

nor full spectrum is available.

%A onoff

Two source names, for the on=1 and on=0 (off) positions. This will

override the use of the "on" uv variable, which is how normal

autocorrelation data are tagged for single dish work.

This mode also disallows usage of Tsys hidden in on=-1 tagged

data.

Default: not used, since on/off tagging is done via the 'on' uv var

%A options

Different computational output options (mainly for debugging).

Exclusively one of the following (minimum match):

'spectrum'     Compute Tsys*(on-off)/off.  This is the default

'difference'   Compute (on-off)

'ratio'        Compute on/off

'on'           Output (on)

'off'          Output (off)

'tsys'         Tsys

%A slop

Allow some fraction of channels bad for accepting

Default: 0

%A debug

Set to TRUE if you truely want to see lots of debugging output.

Default: false

%A repair

A list of bad channels (birdies) that need to be repaired by

interpolating accross them. CLASSy data need 80 for each window

of 159 channels.  eg. repair=80,239,398

Default: not used.

%A mode

Interpolation mode between OFF before and after ON.  Default is 0,

meaning no interpolation done, the last OFF scan is used, or an

average (see OAVER= below) is used.

1 signifies linear interpolation.

** not used **

%A oaver

Avering mode for the OFF. Two integers, denoting the number of

OFF's before the current ON, and the number after.  Use a negative

number to force averaging all scans.  The default is to look at the

most recent one, i.e. oaver=1,0.

%A normalize

Should all scan be normalized before using the OFF?

Using larger oaver= should be combined with normalize=true

Default: false

%A log

Optional filename in which the ON pointing centers (in offset arcsec)

are listed. A third column designates if this pointing center

was the one immediately preceded by an OFF pointing.