%N smooth

%D Convolve an image (in the image domain) with a 2-D gaussian

%P nebk

%: analysis

%B

SMOOTH is a MIRIAD task which convolves an image by an elliptical

gaussian or a boxcar the hard way.   The convolving Gaussian and

boxcar have peaks of unity.  Additional scaling is provided by

the keyword "scale"

By default, SMOOTH will mask pixels in the output image if

there are more masked pixels than unmaked pixels in the

Gaussian convolution area. This means that pixels which were

maked in the input may be unmasked in the output.

See also CONVOL for the fft based method that is generally faster.

HANNING is a program that only smoothes along the velocity axis.

%A in

The input image.  Wild card expansion is supported, no default.

%A out

The output image.  No default.

%A type

Specifies the type of function to convolve by.  Should

be one of

"gaussian"     Gaussian at arbitrary position angle

"boxcar"       Boxcar oriented in x and y directions. Note

that the full width is rounded up to be an

odd number of pixels.

Default is gaussian.  Minimum match is active.

%A fwhm

The Gaussian FWHM along the major and minor axes or the boxcar

full widths in the x and y directions (all in arcseconds).

The image pixel increments are assumed to be in radians.

No default.

%A pa

The position angle in degrees CCW from North of the major axis

of the gaussian.  Not used for boxcar smoothing.

Default is 0.0.

%A scale

If unset, then SMOOTH will attempt to make the units of the

smoothed image be Jy/beam for Gaussian convolution.  If 0.0,

then the convolution integral is scaled (multipled) by the

inverse of the volume of the convolving function. Otherwise,

the integral is scaled by "scale"

%A options

"nocheck"   By default, blanked input pixels do not contribute to the

convolution sum.  If you set NOCHECK then blanked input pixels

are not checked for (but the output image is blanked around the

unconvolved edge, and wherever the input image is blanked).

"force"     Force masking of pixels in the output image which

are masked in the input image.