%N xyauto

%D Solve for LCP-RCP phase difference from grid calibration

%P rlp

%: calibration

%B

Xyauto is a highly specialized program that derives the 'xyphase'

for polarization observations with CARMA.  (For circularly polarized

feeds 'xyphase' is the LCP-RCP phase difference).  The input data

must contain observations of the wire grid polarizers on ants 1-6;

normally these data are chosen using select='purpose(P)'.  Xyauto

generates a phase-only passband solution from LR autocorrelation

spectra.  In this solution the RCP phases are zero for all antennas;

the LCP phases are nonzero only for antennas 1-6.  One should rewrite

the data with uvcat to apply these corrections, then fit the passband

in the usual way with mfcal, making certain to choose one of ants 1-6

as the refant - this will extend the xyphase calibration to all antennas.

%A vis

Input visibility data file. No default. This is expected to

contain multiple channels and spectral windows.

%A line

Standard line parameter, with standard defaults.

%A edge

The number of channels, at the edges of each spectral window, that

are to be dropped. Either one or two numbers can be given, being the

number of channels at the start and end of each spectral window to be

dropped. If only one number is given, then this number of channels

is dropped from both the start and end of each window. The default

value is 0.

%A select

Standard uv selection. Be sure to select only data with the wire

grids in place, usually designated by 'purpose(P)'; no other

selection is needed - xyauto reads only LR autocorrelation data

for ants 1-6.