%N calapply

%D Apply gain and passband corrections producing calibrated data

%P lgm

%: calibration

%B

CALAPPLY applies gain and passband corrections producing an output

calibrated dataset. The input dataset is untouched. Gains and

passband solutions can be applied jointly or independently. Data

outside the valid time range of gains fits may by flagged bad.

%A vis

Input UV dataset(s) for calibration. Although multiple input datasets

are allowed, use only related data that are meant for INVERT to be

mapped. No default.

%A gcal

Name of dataset with gain calibration fits. This must be a dataset

produced by calmake. If supplied, a sideband averaged, but baseline

dependent gain (in Jy/K) is also written to the output dataset.

Default is blank and results in no gain calibration.

%A pcal

Name of dataset with passband calibration fits. This must be a

dataset produced by passmake.

Default is blank and results in no passband calibration.

%A flagbad

Two element true/false parameter for controlling flagging of data

beyond range of validity of amplitude and phase gains fits. If

flagbad=true,true, data outside valid time range of gains

fits will be flagged bad (although the data are still calibrated

using extrapolation). If flagbad=false,false, no data are flagged

bad.

Default is flagbad=true,true.

%A select

Standard uvdata selection. Default is all data.

%A line

Standard line selection. Default is all available channels,

wide and narrow.

Note: there is a problem when input data has only wide band

data.

%A out

Name of output dataset for calibrated data. No default.