%N polargrid

%D Regrid an image to a polar coordinate system

%P pjt

%: map analysis

%B

Sample an image in terms of a polar coordinate system and create

a new image where the x-axis is the azimuthal coordinate and the

y-axis is the radial coordinate.  The radial coordinate can be

in either decimal or logarithmic units.

See also the REGRID program for more WCS based transformations.

Note this program assumes an astronomical image, and square pixels.

%A in

Input data cube.

No default.

%A out

Interpolated regridded output data cube.

No default.

%A center

Center of the polar coordinate system, in pixels where

(1,1) is the lower left point in a map.

By default the mapcenter will be taken.

%A phase

Polar coordinate phase angle (degrees).

Default: 0.0

%A radius

Starting, ending and step value in radial grid.

Optionally these three numbers can be followed by the

coordinate type (a string). Currently supported are 'linear',

'logarithmic' and 'ln' (natural logarithmic).

The units of the radii are in the units of that

coordinate system.

Example: -1,2,0.01,log is a logarithmic axis from 0.1" to 100"

By default the all pixels will fit in the output map, with a

linear grid and stepsize of 1 pixel.

%A angle

Starting, ending and stepsize in polar angle, as measured from

the phase angle given earlier.  If the starting angle is larger

than the ending angle, the coordinate system will be defined

with angle  increasing in the clockwise sense rather than

in a counterclockwise sense.  Angles are given in degrees.

** i believe the program does the reverse **

Default: -180,180,1